3GPP TSG-RAN Meeting #85 	R1-166022

- Nanjing, China, 23rd - 27th May 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | 686 | rev |  | Current version: | 13.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on clarification for channel sensing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, Broadcom, WILUS |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_LAA-Core |  |  |  |  |  |  | Date: |  | 2016-05-13 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | The LS received in R1-163375 recommends clarification to the LAA channel sensing procedure procedure |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that the time quanta of re-attempting sensing the medium is a slot of duration 9 µs instead of Td |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Reduced performance for LAA due to less frequent check of channel availability |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 15.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | This CR has isolated impact |  |  |  |  |  |  |  |  |

### 15.1.1	Channel access procedure for transmission(s) including PDSCH

The eNB may transmit a transmission including PDSCH on a channel on which LAA Scell(s) transmission(s) are performed , after first sensing the channel to be idle during the slot durations of a defer duration Td; and after the counter Nis zero in step 4. The counter N is adjusted by sensing the channel for additional slot duration(s) according to the steps below:

1)	set N=Ninit, where Ninit is a random number uniformly distributed between 0 and CWp, and go to step 4;

2)	if N>0 and the eNB chooses to decrement the counter, set N=N−1;

3)	sense the channel for an additional slot duration, and if the additional slot duration is idle, go to step 4; else, 	go to step 5;

4)	if N=0, stop; else, go to step 2.

5)	sense the channel until either a busy slot is detected within an additional defer duration Td or  all the slots of the additional defer duration Td are detected to be idle

Td

6)	if the channel is sensed to be idle during all the slot durations of the additional defer duration Td, go to step 2; else, go to step 5;

If an eNB has not transmitted a transmission including PDSCH on a channel on which LAA Scell(s) transmission(s) are performed after step 4 in the procedure above, the eNB may transmit a transmission including PDSCH on the channel, after sensing the channel to be idle at least in the slot durations of an additional defer durationTd.

The defer duration Td consists of duration 16us≤Tf≤16us+Tsimmediately followed by mpconsecutive slot durations where each slot duration is 9us≤Tsl≤9us+Ts, and includes an idle slot duration Tslat start of ;

A slot duration Tslis considered to be idle if the eNB senses the channel during the slot duration, and the power detected by the eNB for at least 4uswithin the slot duration is less than energy detection threshold XThresh. Otherwise, the slot duration Tsl is considered to be busy.

CWmin,p≤CWp≤CWmax,pis the contention window. CWpadjustment is described in sub clause 15.1.3.

CWmin,pand are chosen before step 1 of the procedure above.

mp, CWmin,p, and are based on channel access priority class associated with the eNB transmission, as shown in Table 15.1.1-1.

XThresh adjustment is described in sub clause 15.1.4

If the eNB transmits discovery signal transmission(s) not including PDSCH when N>0in the procedure above, the eNB shall not decrement N during the slot duration(s) overlapping with discovery signal transmission.

The eNB shall not continuously transmit on a channel on which the LAA Scell(s) transmission(s) are performed, for a period exceedingTmcot,p as given in Table 15.1.1-1.

For p=3and p=4 , if the absence of any other technology sharing the carrier can be guaranteed on a long term basis (e.g. by level of regulation), Tmcot,p=10ms, otherwise, Tmcot,p=8ms.

========================== Unchanged sections omitted ================================