3GPP TSG RAN WG1 #81	R1-153659

Fukuoka, Japan, 25-29 May 2015

Title:	Response LS related to LAA-802.11 Coexistence

Release:	Release 13

Work Item:	FS_LTE_LAA

Source:	RAN1

To:	IEEE 802 LAN/MAN Standards Committee

Cc:	RAN

Contact Person:

#### Name:	Peter Gaal

E-mail Address:	pgaal@qti.qualcomm.com

Attachments:

1. Overall Description:

3GPP RAN1 would like to thank IEEE 802 LMSC for their LS (R1-152531) “Follow-up Liaison Statement Regarding LAA”, 18 May 2015.

In response, RAN1 would like to provide the following information.

IEEE recommendation 13: It is the technical opinion of IEEE 802 that category 4 LBT provides the best coexistence mechanism for use with 802.11 networks, and hence recommends that it be included as a coexistence mechanism in LAA.

RAN1 response 13: Based on the majority of the evaluation results, both category 3 LBT and category 4 LBT meet the fairness criteria given in [3] in the agreed evaluation scenarios. Note that this does not preclude that other mechanisms can also meet the fairness criteria.

RAN1 would like to further note that RAN1 has made the following agreement:

- It is recommended that a Category 4 LBT mechanism is the baseline at least for LAA DL transmission bursts containing PDSCH.

- It is recommended that LAA supports uplink LBT at the UE. In LAA systems, where the UE’s uplink transmissions are controlled by the eNB, the uplink LBT scheme can be different from the downlink LBT scheme for an LAA SCell.

More information on the agreed Category 4 based LBT mechanisms can be found in Section 7.2.1.6 of [1].

IEEE recommendation 14: IEEE 802 encourages 3GPP RAN1 to continue simulations during the Work Item to investigate and validate design decisions before adopting them into the Technical Specification.

Invitation: To expand collaboration with 3GPP and other industry stakeholders, IEEE 802 proposes working with 3GPP RAN to plan a one day workshop hosted by IEEE 802 on Thursday 16-July, collocated with the July 2015 IEEE 802 Plenary session.

RAN1 response 14: If a follow up Work Item is opened, the specified coexistence mechanisms would be based on the Study Item recommendations. It can be assumed that with the same mechanism being specified in a possible follow up LAA Work Item, the Study Item conclusions on coexistence would remain to be valid.  It can be also expected that further refinements in the specified coexistence mechanisms would be accompanied by corresponding further evaluations in the possible LAA Work Item.

Regarding the notice on the July 2015 IEEE 802 Plenary session, RAN1 would like to note that he possibility of organizing an LAA workshop is being discussed by RAN, therefore RAN1 respectfully defers the related discussion to the RAN plenary for further consideration.

IEEE recommendation Issue 1: 3GPP RAN1 has not yet responded to IEEE 802’s recommendation to consider video traffic scenarios. There are multiple industry predictions that the vast majority of network traffic will be video based within a few years, and today hundreds of millions of unique consumers use real time two-way video calling each month.

RAN1 response Issue 1: RAN1 is not planning to add video traffic to the simulation scenarios analysed during the Study Item phase.  RAN 1 notes that other delay intolerant traffic (VoIP specifically) is an optional part of the evaluation with mixed traffic models which also include delay tolerant traffic such as FTP traffic (see Section A.1 in Annex of [1]).

RAN1 would like to also note that the fact that RAN1 is not planning to add further simulation scenarios during the Study Item phase has not precluded the possibility of interested companies submitting results to address other traffic model cases.

IEEE recommendation Issue 2: IEEE 802 remains interested in understanding coexistence performance under additional high density scenarios, e.g. with 50-200 devices per 802.11 AP, where the impact on mean contention windows and signaling traffic are more significant. These scenarios are typical in environments such as stadiums and dense city areas, which are those where it is likely that IEEE 802.11 and LAA networks will be collocated. The IEEE 802 has included dense scenarios in their work on 802.11ax and believes these would also need to be understood for LAA.

RAN1 response Issue 2: RAN1 is not planning to add further simulation scenarios during the Study Item phase, including addressing even higher density scenarios.

RAN1 would like to also note that the fact that RAN1 is not planning to add further simulation scenarios during the Study Item phase has not precluded the possibility of interested companies submitting results to address denser scenario cases.

It is RAN1s understanding that in a scheduled system, such as LAA, the number of devices simultaneously contending for channel access can be substantially less than the number of devices contending for the channel in an unscheduled system. It is the opinion of some companies in RAN1 that scheduled access can be an effective mechanism to improve coexistence.

RAN1 would also like to point out that RAN1 is not aware of the characteristics of the 802.11ax system being studied in IEEE 802.11, and thus it is difficult for RAN1 to consider evaluations to assess the coexistence of LAA with 802.11ax at this time.

IEEE recommendation Issue 3: 3GPP RAN1 declined to include 3 and 4 Tx/Rx and 80/160MHz configuration in the simulations on the basis that these features are common to LAA and 802.11 and so do not affect coexistence. IEEE 802 agrees with the justification but is concerned the results simulations underestimate the absolute performance of 802.11 systems.

RAN1 response Issue 3: The objective of the LAA Study Item has been to evaluate coexistence between 802.11 and LAA according to the agreed fairness criteria [3]. It has not been an objective of the Study Item to estimate the 802.11 absolute performance or compare the LAA and 802.11 system performance. RAN1 would like to note that, as it was also mentioned in [2], it can be expected that both the 802.11 and LAA systems would benefit from higher number of Tx/Rx antennas and/or other advanced features.

IEEE recommendation Issue 4: In addition, IEEE 802 notes that there have been a small number of recent submissions to 3GPP relating to hidden station issues (e.g. 3GPP R1-151816, R1-151047, R1-151106, R1-151123, and R1-151972). These submissions indicate significant potential for coexistence issues between LAA and 802.11 without detection and/or mitigation mechanisms for both 802.11 hidden stations and LAA hidden stations. IEEE 802 looks forward to further discussion of this issue at the proposed workshop.

RAN1 response Issue 4: RAN1 would like to note that the agreed evaluation scenarios represent cases in which hidden nodes are already present, as was noted, for example, in R1-151816 mentioned above; therefore their impact is already captured in the evaluations. The simulation results in general suggest that applying adaptive Energy Detection (ED) threshold by LAA eNBs can effectively mitigate the hidden node problem and ensure coexistence according to the agreed fairness criteria [3].

RAN1 would like to further note that RAN1 has agreed to include the following in the Technical Report [1]:

- For LAA, it is recommended that LAA supports a mechanism to adaptively change the energy detection threshold, at least for the downlink, i.e., it is recommended that LAA support a mechanism to adaptively lower the energy detection threshold from an upper bound.

In addition, RAN2 has made the following agreement:

- RSSI:

  - For the purpose of detecting hidden node in channel selection UE reporting of RSSI measurements to the eNB is considered useful. 
The details of the RSSI measurement reporting should be discussed in stage-3.

  - The eNB indicates which carriers(s) the UE should report RSSI for.

Further discussion: IEEE 802 looks forward to further discussions with 3GPP on how to engage all stakeholders in the review and development of LAA coexistence mechanisms

The 3GPP RAN1 response noted that it was deferring IEEE 802’s questions related to the inclusion of other stakeholders into the LAA development and review process to 3GPP RAN. IEEE 802 proposes expanded collaboration with 3GPP and other industry stakeholders, by working with 3GPP RAN to plan a one day workshop hosted by IEEE 802 on Thursday 16 July, collocated with the July 2015 the IEEE 802 Plenary session at the Hilton Waikoloa Village in Hawaii.

RAN1 response: RAN1 thanks IEEE 802 LMSC for the notice. The possibility of organizing an LAA workshop is being discussed by RAN, therefore RAN1 respectfully defers further answers to this discussion to RAN.

2. Actions:
To IEEE 802 LMSC
ACTION:   RAN1 would like to respectfully ask IEEE 802 LMSC to take the above information into account.

## 3. References

[1] TR 36.889, “Study on Licensed-Assisted Access to Unlicensed Spectrum (Release 13)”

[2] R1-151381, “Response LS on LAA-802.11 Coexistence”, RAN1

[3] RP-141817, "Revised SID: Study on Licensed-Assisted Access to Unlicensed Spectrum"

4. Date of Next TSG-RAN WG1 Meetings:

TSG RAN WG1 Meeting 82	24 - 28 August 2015	Beijing, China

TSG RAN WG1 Meeting 82bis	5 - 9 October 2015	Malmo, Sweden