| MAN/WAN Technologies |  |  |  |  | LAN Technologies |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T1 1.5Mbps | DS3 45Mbps | OC3 155 Mbps | OC12 622 Mbps | OC192 10 Gbps | Ethernet 10 Mbps | Ethernet 100 Mbps | Ethernet 1 Gbps |
| 8ms | 0.26ms | 0.077 ms | 0.019 ms | .0012 ms | 1.2ms | 0.12ms | .012ms |

Figure 1: Maximum Transmission Unit Transmission times (1.5 Kbytes)  at varios link speeds. Worst case jitter (J1) for voice caused by waiting for lower priority class packet.

| Voice flows | MAN/WAN Technologies |  |  |  |  | LAN Technologies |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | T1 1.5Mbps | DS3 45Mbps | OC3 155 Mbps | OC12 622 Mbps | OC192 10 Gbps | Ethernet 10 Mbps | Ethernet 100 Mbps | Ethernet 1 Gbps |
| 1 | 0.36ms | 0.012ms | 0.0035 ms | 0.00087ms | 0.00005ms | 0.054ms | 0.005ms | 0.0005ms |
| 10 | 3.6ms | 0.12ms | 0.035ms | 0.0087ms | 0.0005 ms | 0.544ms | 0.054ms | 0.005ms |
| 100 | * | 1.2ms | 0.35ms | 0.087ms | 0.0054ms | 5.4ms | 0.544ms | 0.054ms |
| 1000 | * | * | 3.5ms | 0.87ms | 0.0544ms | * | 5.4ms | 0.544ms |
| 10000 | * | * | * | 8.7ms | 0.544ms | * | * | 5.4ms |

Figure 2: Worst case jitter (J2) for voice caused by waiting for packets in same class (EF) at various link speeds. * indicates aggregate voice flow bandwidth exceeded 50% of link capacity.