3GPP TSG-RAN WG1 Meeting #84bis 	R1-163436

Busan, Korea, 11th - 15th April 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | 0626 | rev | _ | Current version: | 13.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on channel access procedure after an additional defer duration for DL LBT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | WILUS Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_LAA-Core |  |  |  |  |  |  | Date: |  | 2016-04-11 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | According to the current specification, when a channel is busy during random back-off duration, there is no difference on the number of sensing slots for random back-off between Ninit=1 and Ninit=2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modify channel access procedure in clause 15.1.1 to clarify channel access procedure in case of counter N=0 after the channel is sensed to be idle during the slot durations of the additional defer duration Td. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | When a channel is busy during random back-off duration, there is no difference on the number of sensing slots for random back-off between Ninit=1 and Ninit=2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 15.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | This CR has isolated impact. |  |  |  |  |  |  |  |  |

### 15.1.1	Channel access procedure for transmission(s) including PDSCH

The eNB may transmit a transmission including PDSCH on a channel on which LAA Scell(s) transmission(s) are performed , after first sensing the channel to be idle during the slot durations of a defer duration Td; and after the counter Nis zero in step 4. The counter N is adjusted by sensing the channel for additional slot duration(s) according to the steps below:

1)	set N=Ninit, where Ninit is a random number uniformly distributed between 0 and CWp, and go to step 4;

2)	if N>0 and the eNB chooses to decrement the counter, set N=N−1;

3)	sense the channel for an additional slot duration, and if the additional slot duration is idle, go to step 4; else, 	go to step 5;

4)	if N=0, stop; else, go to step 2.

5)	sense the channel during the slot durations of an additional defer duration Td;

6)	if the channel is sensed to be idle during the slot durations of the additional defer duration Td, go to step 4; else, go to step 5;

If an eNB has not transmitted a transmission including PDSCH on a channel on which LAA Scell(s) transmission(s) are performed after step 4 in the procedure above, the eNB may transmit a transmission including PDSCH on the channel, after sensing the channel to be idle at least in the slot durations of an additional defer durationTd.

The defer duration Td consists of duration 16us≤Tf≤16us+Tsimmediately followed by mpconsecutive slot durations where each slot duration is 9us≤Tsl≤9us+Ts, and includes an idle slot duration Tslat start of ;

A slot duration Tslis considered to be idle if the eNB senses the channel during the slot duration, and the power detected by the eNB for at least 4uswithin the slot duration is less than energy detection threshold XThresh. Otherwise, the slot duration Tsl is considered to be busy.

CWmin,p≤CWp≤CWmax,pis the contention window. CWpadjustment is described in sub clause 15.1.3.

CWmin,pand are chosen before step 1 of the procedure above.

mp, CWmin,p, and are based on channel access priority class associated with the eNB transmission, as shown in Table 15.1.1-1.

XThresh adjustment is described in sub clause 15.1.4

If the eNB transmits discovery signal transmission(s) not including PDSCH when N>0in the procedure above, the eNB shall not decrement N during the slot duration(s) overlapping with discovery signal transmission.

The eNB shall not continuously transmit on a channel on which the LAA Scell(s) transmission(s) are performed, for a period exceedingTmcot,p as given in Table 15.1.1-1.

For p=3and p=4 , if the absence of any other technology sharing the carrier can be guaranteed on a long term basis (e.g. by level of regulation), Tmcot,p=10ms, otherwise, Tmcot,p=8ms.

Table 15.1.1-1: Channel Access Priority Class

| Channel Access Priority Class (p) | mp | CWmin,p | CWmax,p | Tmcot,p | allowed CWpsizes |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | 3 | 7 | 2 ms | {3,7} |
| 2 | 1 | 7 | 15 | 3 ms | {7,15} |
| 3 | 3 | 15 | 63 | 8 or 10 ms | {15,31,63} |
| 4 | 7 | 15 | 1023 | 8 or 10 ms | {15,31,63,127,255,511,1023} |

For LAA operation in Japan, if the eNB has transmitted a transmission after N=0in step 4 of the procedure above, the eNB may transmit the next continuous transmission, for duration of maximum =4 msec, immediately after sensing the channel to be idle for at least a sensing interval of =34usec and if the total sensing and transmission time is not more than 1000⋅Tmcot+Tmcot  Tj−1⋅Tjs µsec. consists of duration Tf=16usimmediately followed by two slot durations  each and includes an idle slot duration Tslat start of . The channel is considered to be idle for Tjsif it is sensed to be idle during the during the slot durations of Tjs.