3GPP TSG RAN WG1 #87	R1-1613770

November 14nd – 18th 2016

Reno, USA

Title:	Response LS to IEEE 802.11 regarding LAA

Release:	Release 13

Work Item:	LTE_LAA

Source:	RAN1

To:	IEEE 802 LAN/MAN Standards Committee, RAN

Cc:	Wi-Fi Alliance, The City of New York Office of the Mayor, RAN2, RAN4

Contact Person:

#### Name:	Srinivas Yerramalli

E-mail Address:	syerrama@qti.qualcomm.com

Attachments:

1. Overall Description:

3GPP RAN1 would like to thank IEEE 802 LMSC for their LS (R1-167908) “Liaison Statement to 3GPP from IEEE 802 LMSC” and LS (R1-1613126) “IEEE 802 LMSC response LS on energy detection threshold in future 802.11 technologies”.

In response, RAN1 would like to provide the following information.

IEEE Recommendation 1-1:

IEEE 802’s Liaison Statement to 3GPP RAN1 dated 18 March 2016 (IEEE 802 19-16-0037-09-0000-laa-comments.pdf ) suggested in comment 1 that LAA should be modified to avoid sending energy for the primary purpose of blocking access to the channel to others. IEEE 802 documented two options for potential modifications. The basis for IEEE 802’s suggestions was its understanding that LAA needs to maintain control of medium between gaining access and transmitting synchronized data bursts by sending energy, but transmitting energy for sole purpose of blocking others is contrary to best practice everywhere and possibly regulations in some domains.

3GPP RAN1’s response to this comment was included in 3GPP RAN1’s Liaison Statement dated 7 June 2016 (R1-166041). 3GPP RAN1 noted that LAA Rel. 13 does not mandate transmitting any signals between the time the channel access is obtained and the subframe or slot boundary and that any such signals are an implementation choice and not a matter for specification in LAA Rel. 13. 3GPP RAN1 provided four reasons why no changes are required in LAA Rel. 13 to address IEEE 802’s concerns.

3GPP RAN1’s response was discussed at a recent ETSI BRAN based on a submission (BRAN(16)000111r0) that undertook a detailed decomposition of 3GPP RAN1’s response. This submission proposed a regulatory action in Europe that would ban the transmission of signals for the primary purpose of blocking other devices from using a channel. The basis of this proposal was that the four reasons provided by 3GPP RAN1 are either invalid or irrelevant to a conclusion that no change was necessary to LAA Rel. 13. Instead, it was asserted in the submission that the four reasons actually provide excellent support for a conclusion that LAA Rel. 13 systems should not transmit signals to reserve the channel between the time they gain access to a channel using the LBT mechanism and the time they are ready to transmit LAA Rel. 13 defined signals.

It is worth noting that that the discussion on this proposal at the ETSI BRAN meeting was very contentious, with stakeholders from both the 3GPP and IEEE 802 communities participating, and that there was no consensus for the proposal for regulatory action. No doubt the issue will be discussed within ETSI BRAN again in the near future.

However, there was agreement at the ETSI BRAN meeting that regardless of what was regulated in Europe or included by 3GPP RAN1 in LAA Rel. 13, LAA Rel. 13 systems operating in Europe must satisfy RE-Directive 2014/53/EU, which requires radio equipment to make efficient use of spectrum and avoid harmful interference. In particular, this Directive:

- References article 2.2 (1) in Directive 2002/21/EC, which states in part that ‘radio equipment’ means an electrical or electronic product, which intentionally emits and/or receives radio waves for the purpose of radio communication and/or radiodetermination …

- States in article 3.2 that radio equipment shall be so constructed that it both effectively uses and supports the efficient use of radio spectrum in order to avoid harmful interference

A case was made at the ETSI BRAN that that an LAA device transmitting signals primarily for the purpose of blocking other devices from accessing a channel would not satisfy the RE-Directive 2014/53/EU’s definition of radio equipment. The specific problem was asserted to be that radio communications inherently requires both a transmitter and a receiver. A receiver function is missing in this case because no device needs to demodulate the signal that blocks other devices using the channel, noting that the LAA Rel. 13 does not contain any specification for the transmission or reception of such signals.

It was also asserted at the ETSI BRAN meeting that transmitting signals primarily for the purpose of blocking other devices from accessing the channel is contrary to the rule in Europe requiring efficient use of the medium because such transmissions make use of the channel but are actually unnecessary. The unnecessary nature of any such transmissions was highlighted by the 3GPP RAN1 Liaison Statement, which in the first reason related to comment 1 noted that fair coexistence between LAA and Wi-Fi was possible, and that LAA could operate with good performance, without the transmission of such signals. 3GPP RAN1’s Liaison Statement specifically states in this regard that:

...deferring sending energy until a subframe boundary or partial subframe boundary, satisfied the criteria that the presence of an LAA network doesn’t cause more degradation to 802.11 than the presence of another 802.11 network, and also provided good LAA performance, so it is considered a viable implementation option

It needs to be noted the discussion at the most recent ETSI BRAN meeting related to how the RE-Directive 2014/53/EU’s rules should be interpreted in the context of the transmission of signals that block access to a channel was also very contentious and there was no consensus.

At this time, many IEEE 802 stakeholders believe that the issues highlighted in comment 1 in IEEE 802’s Liaison Statement to 3GPP RAN1 dated 18 March 2016 remain unresolved. However, 3GPP RAN1’s response to comment 1 suggests an opportunity for an acceptable compromise that should satisfy the needs of all stakeholders. The key insight highlighted by 3GPP RAN1 in their response is that deferring sending energy until a subframe boundary or partial subframe boundary does not cause any harm to the performance of LAA and supports fair sharing between LAA and Wi-Fi systems.

3GPP RAN1 is urged to seriously consider and accept the following IEEE 802 proposal to assist the closure of an issue that is very important to many IEEE 802 stakeholders.

This insight suggests that consensus can be achieved between all stakeholders. It is now recommended that 3GPP RAN1 include something similar to the following note in the LAA Rel. 13 specification:

Implementation note: LAA devices should not transmit any signals in a channel between the time a device obtains access to the channel using LBT Category 4 and the time of the next subframe or partial subframe boundary because transmitting such signals:

- May violate rules in some regulatory domains

- Is contrary to the best practice that unnecessary transmissions are avoided in unlicensed spectrum

- Is unnecessary because LAA can achieve good performance without such signals

- Is unnecessary because LAA can achieve fair sharing with other technologies without such signals

An acceptable alternative would be for 3GPP RAN1 to agree publically with the principles articulated by the proposed note. Under both alternatives, enforcement would need to be left to other authorities.

RAN1 response 1-1:

RAN1 is aware of the discussions in ETSI BRAN regarding the cited proposals as well as the interpretation of the regulatory requirements. RAN1 views the transmission of signals transmitted between the time a device obtains access to the channel using LBT Category 4 and the time of the next subframe or partial subframe boundary as overhead. Any system has a many different forms of overhead.  While minimization of overhead is a goal, there are benefits to knowing the timing of transmitted signals at the receiver and hence the overhead incurred in this case can yield improvements in system performance. While RAN1 does not specify the transmission of any such signals, RAN1 recognizes the benefits of leaving the flexibility and choice of transmission of such signals as an implementation and operational choice. Furthermore, numerous evaluations have included the use of such signals and showed that good coexistence with other devices operating in the same channel can be achieved even with the presence of such signals. Finally, it is noted that nodes of another technology will not understand the signal that was transmitted before the subframe or partial subframe boundary irrespective of the nature of the signal transmitted by an LAA eNB. Therefore RAN1 does not believe that any restrictions on nodes should be imposed. However, in the interest of serving the general goal of minimizing overhead, RAN1 is discussing adding a statement in 36.300 to minimize the transmission of such signals. However, no consensus has been reached yet on the specific wording of the statement.

IEEE Recommendation 1-2: IEEE 802 suggests an additional solution based on the definition of multiple sub-frame starting positions

IEEE 802’s Liaison Statement to 3GPP RAN1 dated 18 March 2016 (IEEE 802 19-16-0037-09-0000-laa-comments.pdf ) suggested in comment 1 that LAA should be modified to avoid sending energy for the primary purpose of blocking access to the channel to others. IEEE 802 provided two possible solutions for consideration by 3GPP RAN1 to resolve this problem. However, it has since been noted by an IEEE 802 participant that there is at least one additional solution that might be acceptable to all parties.

The goal of any solution is to minimise the time from when LAA gains access to the channel and the time it starts transmission of a sub-frame or partial sub-frame. Currently, LAA only supports two starting positions (0.5ms and 1ms boundaries i.e. 6/14 OFDM symbols) , which results in a difference between the time from when LAA gains access to the channel and the time it starts transmission of a sub-frame or partial sub-frame of up to 0.5ms or 1ms. It is contrary to regulations and/or best practice for an LAA system to transmit energy in such a long gap for the primary purpose of reserving the channel.

The additional possible solution is to reduce the duration of any reservation signal by increasing the number of possible starting positions of a partial sub-frame. IEEE 802 recognises that this may lead to increased complexity for the LAA eNB because LAA, not knowing in advance when the LBT may be successful, may need to keep Transport Blocks (TBs) compatible with all possible lengths of a partial sub-frame prepared in advance. However, doing so would just be a logical extension of what has to be done for two starting positions and will be worthwhile because it reduces the wastage of unlicensed resources.

IEEE 802 requests that 3GPP RAN consider defining additional partial sub-frame starting positions in LAA Rel. 13, so that the need to send reservation signals is minimized. IEEE 802 notes that 3GPP RAN is already considering shortened sub-frames (see RP-161299) because they help reduce latency in LTE and adapt it better for time critical applications (gaming, V2V and V2X communications etc.).

Further, IEEE 802 requests 3GPP RAN1 specify the use of this feature as a mandatory requirement for the eNB in LAA Rel. 13 rather than leaving it as an eNB recommendation as is the case currently.

RAN1 response 1-2:

RAN1 considered various aspects when designing the partial subframes to be specified as part of LAA including the amount of channel occupancy time that is not utilized for data transmission, eNB complexity in preparing various TBs, UE complexity to process partial subframes, the efficiency of HARQ combining with many different partial subframe sizes, UE power consumption impact due to higher frequency of channel monitoring and UE processing timelines needed to detect a transmission (including CRS detection, (E)PDCCH detection). Taking these various factors into account, it was agreed to have 2 start positions for the start of transmission and 6 end partial subframe configurations for Rel-13 LAA.

RAN1 further notes that 2-symbol, 4-symbol and 1-slot TTI are being considered in Rel-14 as part of [1] for frame structure type 1 and type 2 (FS1 and FS2) and shortened TTI candidates may be incorporated into FS3 for LAA in the future if and when RAN plenary approves the corresponding work item (see [3] for a candidate proposal that was discussed in RAN-73). When specified, the short TTIs may enable the eNB and the UEs to reduce the time between channel access and the beginning of LTE transmissions.

IEEE Recommendation 1-3: IEEE 802 requests 3GPP RAN1 confirm that HARQ operation is not related to comment 1

IEEE 802’s Liaison Statement to 3GPP RAN1 dated 18 March 2016 (IEEE 802 19-16-0037-09-0000-laa-comments.pdf ) suggested in comment 1 that LAA should be modified to avoid sending energy for the primary purpose of blocking access to the channel to others. Part of 3GPP RAN1’s response to this comment included the following assertion, which IEEE 802 believes is incorrect:

The proposed changes would have an effect in making HARQ operation less efficient, since first transmission and retransmission would use different subframe lengths

IEEE 802 notes the following extract from 3GPP TS 36.321:

If the MAC entity receives a retransmission with a TB size different from the last valid TB size signalled for this TB, the UE behavior is left up to UE implementation.

In the context of this extract, IEEE 802 believes 3GPP RAN1 is asserting that if transmissions can happen in multiple time units (and not necessarily in one sub-frame), it is possible that the eNB will not be able to allocate the same TB size (the PHY data packet) for a first transmission and a retransmission. It appears 3GPP RAN1 is claiming that since the UE HARQ behaviour is left undefined, the UE may ignore the retransmission and hence the HARQ gain will be reduced.

However, IEEE 802 notes that different transmission time units don’t necessarily force the eNB to use different TB sizes for retransmissions. HARQ efficiency depends on the TB size in bits and not on the transmission duration in time. The TB size depends upon the MCS used, the number of Physical Resource Blocks (PRBs or frequency resources) allocated and additionally (in case of partial sub-frames) the number of symbols in the subframe. It is possible to maintain the same TB size by adjusting these 3 parameters.

There may be cases when the eNB scheduler is not able to accommodate a retransmission in a certain partial subframe. In that case, the retransmission can be accommodated in a non-partial subframe and the partial subframe can be used for transmitting new data. Also, for retransmitting data that was initially transmitted in a partial subframe, it is possible to adjust number of PRBs and MCS and accommodate it in a non-partial subframe or a partial subframe of a different length.

IEEE 802 requests that 3GPP RAN1 clarifies IEEE 802’s understanding in this case. IEEE 802 further requests that 3GPP RAN1 confirms that HARQ efficiency is not related to the issue raised in comment 1.

RAN1 response 1-3:

In general, RAN1 views that the different transmission time units don’t necessarily force the eNB to use different TB sizes for retransmissions and confirms that TB size is the one that matters for HARQ efficiency. However, RAN1 also notes that given the quantization of the TBS table, the granularity of RB allocation and the scaling factor to be used when using partial subframes etc. may impact the probability of finding a TB size match between the first transmission and a retransmission if initial and end partial subframes are used extensively.

IEEE recommendation 2: IEEE 802 requests 3GPP RAN1 impose additional constraints on DRS overheads

IEEE 802’s Liaison Statement to 3GPP RAN1 dated 18 March 2016 (IEEE 802 19-16-0037-09-0000-laa-comments.pdf ) suggested in comment 2 that the LAA specification be modified to include reasonable limits on how often the channel may be accessed using the DRS mechanism.

3GPP RAN1’s response to this comment was included in 3GPP RAN1’s Liaison Statement dated 7 June 2016 (R1-166041). 3GPP RAN1 agreed to limit the DRS overhead per eNB to 5%.

However, a DRS limit of 5% is still much higher than the typical transmission overhead of equivalent 802.11 transmissions. In 802.11, only the Traffic Indication Map (TIM) and Channel Switch Announcement (CSA) messages can be transmitted with 25us LBT similar to LAA DRS. The transmission of such messages is typically significantly less than 1% per AP.

IEEE 802 requests 3GPP RAN1 further reduce the DRS overhead for LAA Rel. 13 to a value closer to that resulting from 802.11 TIM and CSA overhead, which is significantly less than 1%. Alternatively, IEEE 802 requests that LAA Rel. 13 be specified to use 25us LBT for DRS transmissions up to 1% overhead and Category 4 LBT with EDCA parameters of AC_VO for DRS transmissions beyond 1%. IEEE 802 believes such a change will promote fair access to the medium for both LAA Rel. 13 and IEEE 802.11 systems.

RAN1 response 2:

RAN1 notes that the limit agreed in 3GPP is consistent with the current and proposed ETSI BRAN specification (Section 4.2.7.3.3 of [2]) and wishes to emphasize that it is only a strong upper bound for the amount of time to be used with 25us LBT.

RAN1 also notes that there are two mechanisms using which DRS can be transmitted (1) Using 25us LBT but not multiplexed with PDSCH  and (2) Using Cat 4 LBT when transmitted as part of a regular transmission burst. The Cat 4 LBT to be used would be a function of the channel access priority class of the traffic which is being carried as part of the transmission burst and can be one of the 4 LBT priority classes defined in the 3GPP specification. Thus, LAA already has a mechanism to allow DRS transmission by performing a Cat 4 LBT with one of several different priority classes (and not necessarily the highest priority class as suggested by IEEE 802). Thus, an eNB with traffic will most likely perform a Cat 4 LBT when transmitting DRS to enable multiplexing with PDSCH in the same transmission and the amount of the time the 25us LBT would be used for transmitting DRS would be limited in practice.

Furthermore, it is technically sound for the eNBs at least of the same operator in a given area to coordinate their DRS transmissions within a small window so that UEs may be able to measure and report other serving cells in the vicinity. This strongly limits the incentive for the eNBs of the same operator to use multiple DRS transmissions which are significantly spaced apart in time. Thus, medium occupancy would be minimal overall when observed from a network perspective which is especially beneficial in dense deployments with a lot of eNBs.

IEEE recommendation 3: IEEE 802 requests 3GPP RAN1 to consider the ED threshold based on a more realistic channel model

IEEE 802’s Liaison Statement to 3GPP RAN1 dated 18 March 2016 (IEEE 802 19-16-0037-09-0000-laa-comments.pdf ) suggested in comment 3 that the LAA specification be modified so that LAA system detect 802.11 networks with a similar level of sensitivity to that with which current 802.11 devices can detect each other. Alternatively, IEEE 802 requested that LAA Rel. 13 require a base energy detection threshold of TH = -77dBm (20MHz), or preferably lower.

3GPP RAN1’s response to this comment was included in 3GPP RAN1’s Liaison Statement dated 7 June 2016 (R1-166041). The response rejected IEEE 802’s request based on 3GPP RAN1’s opinion that the agreed threshold levels will ensure fair coexistence as simulations based on the 3GPP indoor scenario have shown fair-coexistence when using the agreed CCA threshold.

However, measurements of deployed 802.11 networks (such as R1-165927 and R1-162982 presented to 3GPP RAN1) show that in many deployments, the median RSSI over all Wi-Fi links is much lower than the median RSSI (=-48dBm) over all links in the 3GPP indoor model. Hence, any fair coexistence evaluations based on the 3GPP indoor model are not directly applicable to coexistence between LAA and 802.11 in deployments with low RSSI 802.11 links. The basis for 3GPP RAN1’s response to IEEE 802 is therefore invalid.

IEEE 802 requests that 3GPP RAN1 consider the fairness of an LAA ED threshold of -72dBm on coexistence between LAA and 802.11 in a configuration that has a larger percentage of weak 802.11 links than what is currently assumed in the 3GPP indoor model. IEEE 802 suggests using the measurements from R1-165927 and R1-162982 as a basis for selection of simulation parameters IEEE 802 would prefer that this issue is resolved before any coexistence issues occur.

RAN1 response 3:

RAN1 re-iterates that the current LBT level of -72dBm was agreed in 3GPP after considerable debate and with wide participation of stakeholders of both LAA and IEEE 802.11 technologies. The discussions included consideration of proposals to transmit and receive IEEE 802.11 preambles. Evaluations in RAN1 considered both indoor and outdoor scenarios where the outdoor scenarios modeled links with lower RSSI than the indoor scenario. RAN1 also notes that Rel-14 eLAA has also incorporated a default ED threshold level of -72dBm for a 23dBm transmit power capable UE when operating with other coexisting technologies in the same spectrum in the absence of any signaling received from the eNB to use a different threshold. In addition, RAN1 has also agreed on RRC signalling which enables the eNB to configure the ED threshold to be used at the UE including values below the ED threshold level of -72dBm. The feasibility of specific values of ED threshold below -72dBm will be studied further in RAN4.

RAN1 also notes that ETSI BRAN with the active participation of all stakeholders including regulatory bodies of several countries has also agreed to an ED threshold level -72dBm in its current draft version of the standard [2]. Furthermore, RAN1 notes that there is already a large asymmetry between threshold level of -62dBm which current IEEE 802.11 devices use to detect LAA and the threshold level of -72dBm which LAA eNBs use to detect IEEE 802.11 devices. It was seen undesirable to further widen this asymmetry.

RAN1 would also like to highlight that RAN4 has decided on the development of a set of coexistence test cases including multi-node tests to verify the coexistence between LAA and IEEE 802.11 devices in various scenarios including testing above and below an ED threshold of -72dBm for LAA devices. While the coexistence mechanisms defined in LAA are based on energy detection, the implementation of other mechanisms by a device is not precluded. LAA equipment would be tested to ensure fair coexistence between LAA and 802.11 systems.

IEEE recommendation 4: IEEE 802 requests that 3GPP RAN1 align LAA & 802.11 slots to preserve efficiency

IEEE 802’s Liaison Statement to 3GPP RAN1 dated 18 March 2016 (IEEE 802.19-16-0037-09-0000-laa-comments.pdf) observed in comment 4 that the LAA specification does not ensure time alignment of its slot boundary with IEEE 802.11 slot boundary, with the effect that large slot time offsets between LAA and 802.11 introduces more transmission collisions which reduce spectral efficiency and degrade both LAA and 802.11 performance. IEEE 802 suggested that LAA should time align its slot boundary with 802.11 slot boundary as accurately as possible. 3GPP RAN1’s response to this comment was included in 3GPP RAN1’s Liaison Statement dated 7 June 2016 (R1-166041).

The first part of 3GPP RAN1’s response focused on IEEE 802’s proposed solution. The response noted that 3GPP RAN1 believes that it would be (an) undue burden to require all LAA equipment to detect 802.11 PHY preamble and MAC NAV field. IEEE 802 notes that it did not actually ask for LAA Rel. 13 to detect the MAC NAV field. Rather, it asked for LAA Rel. 13 to detect and transmit 802.11 PHY preambles. IEEE 802 agrees that this would be an additional burden but it has the benefit of enabling the alignment of LAA and 802.11 slots, which will enhance spectral efficiency. As an alternative, IEEE 802 suggested that LAA Rel. 13 include an improved energy detection mechanism that is more accurately able to detect the boundary at the end of an IEEE 802.11 transmission.

The second part of the response focused on the problem. 3GPP RAN1 asserted that the problem does not occur very often because contending nodes using random backoff based LBT typically do not attempt data transmission within a few microseconds of each other. This response is incorrect and represents a fundamental misunderstanding of the importance of slot synchronization in any LBT system. If LAA Rel. 13 has poor slot synchronization then the highlighted problem is guaranteed to occur one hundred percent of the time. The effect is that slotted ALOHA style access is converted into ALOHA style access; the efficiency of ALOHA is half of slotted ALOHA.

3GPP RAN1 also asserted that there is no problem because the simulations during the SI showed that the presence of an LAA network doesn’t cause more degradation to 802.11 than the presence of another 802.11 network. IEEE 802 agrees that the SI simulations were very useful to show that an LBT Category 4 style scheme was the most appropriate access mechanism for LAA Rel. 13. However, the SI simulations cannot reasonably be used to draw any conclusions about the details of LAA Rel. 13 coexistence with IEEE 802.11 because:

- None of the SI simulations implemented the access method specified in LAA Rel. 13, except in very broad terms, and it is unclear how accurately they implemented the detail of IEEE 802.11 (or what version – both 802.11n and 802.11ac should have been simulated in recognition of the large number of currently deployed systems)

- The SI simulation scenarios were very limited in scope (e.g., the indoor scenario was a simple 4x2 matrix of APs), thus not representing the wide diversity of scenarios found in the real world

- Work presented to ETSI BRAN has established that the propagation models used in the 3GPP evaluations do not simulate common deployment scenarios sufficiently well

3GPP RAN1 finally asserted that the problem could be solved by relying on LBT backoff. IEEE 802 agrees that the LBT backoff mechanism will allow data to be transmitted eventually. However, the downside of a solution that more heavily relies on collisions and backoff is significantly reduced spectrum efficiency and quality of service with a consequent adverse effect on all users of the spectrum.

RAN1 response 4:

RAN1 notes that even detecting 802.11 PHY preambles requires LAA eNBs to implement several aspects of the 802.11 receiver (including at least sampling frequency matching, time and frequency synchronization, packet presence detection and L-SIG processing). RAN1 also notes that, in the presence of hidden nodes, detecting 802.11 PHY preambles would not be sufficient to provide slot alignment. In such scenarios, the MAC header needs to be decoded to determine the NAV duration. This could represent an undue burden on the eNB as it requires the eNB to implement a good fraction of the PHY and MAC layer processing needed to detect the transmissions of IEEE 802.11 devices. Finally, RAN1 also notes that the proposed scheme does not work unless it is mandatory for all technologies coexisting in this spectrum to transmit an 802.11 preamble (and containing a NAV message) before the start of their own transmission, which is not a technology neutral solution.

In addition, RAN1 notes that a smaller energy detection granularity has its own advantagesand disadvantages. For example, the accuracy of measurement may degrade when a smaller granularity of measurement is used. Furthermore, requiring the sensing time in each slot to be 7us (for example) may reduce the amount of time needed for Rx-Tx turnaround and power ramping.  This impacts the LBT procedure from a network perspective and may impose infeasible performance requirements on LAA devices.

RAN1 does not agree with the notion that simulations in 3GPP only represent the channel access scheme in broad terms. RAN1 notes that at least the simulations done as part of the work item for standardizing eLAA in Rel-14 and for contributions to ETSI BRAN have considered the channel access scheme in standardized in 3GPP Rel-13. RAN1 notes that simulations have been comprehensively performed as part of the SI, WI, the follow up WI in Rel-14 and also for ETSI BRAN contributions in a range of scenarios including indoor and outdoor deployments, with and without RTS/CTS, single and multiple channel scenarios, DL only and with varying ratios of DL and UL traffic, with and without VoIP traffic etc. and no coexistence concerns were identified which could be attributed to slot misalignment.

Furthermore, RAN1 would also like to clarify that 3GPP propagation models are compatible with ITU recommendations and have been extensively used for more than a decade. Finally, no evidence has been presented to RAN1 about the deficiency of the evaluation methodology and feedback received from ETSI BRAN participants in RAN1 contradicts IEEE 802’s assertion that any such deficiency of channel modelling has been concretely established in ETSI BRAN.

IEEE recommendation 5: IEEE 802 suggests resolution of the multi-channel aggregation issue be postponed until coexistence tests can be run

IEEE 802’s Liaison Statement to 3GPP RAN1 dated 18 March 2016 (IEEE 802.19-16-0037-09-0000-laa-comments.pdf) observed in comment 5 that non-contiguous and/or differently aligned use of spectrum causes each LAA eNB to impact multiple 802.11 networks. IEEE 802 suggested that LAA should align its multi-channel aggregation scheme with 802.11.

3GPP RAN1’s response to this comment was included in 3GPP RAN1’s Liaison Statement dated 7 June 2016 (R1-166041). The response rejected IEEE 802’s request on the basis that its adoption would reduce 802.11, as well as LAA, performance.

IEEE 802 disagrees with 3GPP RAN1’s assertion that the LAA multi-carrier scheme will not adversely affect fair channel access probabilities for 802.11. This is because the 802.11 multi-carrier schemes follows channel bonding rules, while the LAA multi-carrier scheme can flexibly select any group of carriers for transmission. This additional channel access flexibility for LAA means that in certain multi-carrier configurations, LAA will gain higher channel access at the expense of co-channel 802.11. This has also been shown by simulations presented in 3GPP RAN1 (R1-160816, R1-157009, R1-155547) and ETSI-BRAN (BRAN(15)000188r3).

IEEE 802 believes there is not yet consensus in relation to this issue. However, it is not clear how consensus can be achieved at this time. IEEE 802 suggests that resolution of this issue be postponed until coexistence tests with real equipment can be run to demonstrate any adverse impact on 802.11. Any further comments on this issue by IEEE 802 are likely to be in the context of LAA Rel. 14.

RAN1 response 5:

RAN1 notes that in multi-carrier access schemes that perform independent LBT per carrier, the whole Cat 4 LBT procedure has to be completed on each carrier before any given transmission and hence in principle each carrier waits for its fair share of time determined by congestion and collisions on that carrier before it accesses the medium.

For UEs performing multi-carrier transmission on the uplink, it was also agreed that a UE that has received UL grants on a set of carriers scheduled with Cat 4 LBT with the same starting point in a subframe on all carriers can switch to 25us LBT, if Cat 4 LBT has been performed on a designated carrier in the set of carriers, where the set of carriers is specified according to the ETSI channel bonding rules.

RAN1 notes IEEE 802’s comments and looks forward to any other input that IEEE 802.11 may have on this item.

IEEE recommendation 6: IEEE 802 requests confirmation that it is mandatory to end transmissions at the shortest subframe boundary and further enhancements in LAA Rel. 14.

Based on 3GPP RAN1’s response, it is now IEEE 802’s understanding that LAA Rel. 13 already supports beginning sub-frames with 6/14 OFDM symbols and end sub-frames with 3/6/9/10/11/12/14 OFDM symbols, where each symbol has a duration of approximately 71μs. IEEE 802’s request to 3GPP RAN is that LAA occupy the channel for the minimum time required to transmit data corresponding to the access channel priority class (or higher) used to win channel access and that LAA end the transmission as soon as possible (i.e. at the nearest partial subframe boundary) in the case there is no more data to transmit. IEEE 802 requests that 3GPP RAN1 confirm that LAA Rel. 13 systems are mandatorily required to end transmission at the shortest end partial sub-frame boundary when it has no more data to transmit of the appropriate channel access priority class(s).

Further, IEEE 802 notes that the minimum partial sub-frame duration in LAA Rel. 13 is 6 OFDM symbols for the beginning partial sub-frame and 3 OFDM symbols for the end partial sub-frame. This duration is approximately equal to 426μs and 213μs respectively which is about 106 or 53 times higher than a 4μs symbol quantum in which 802.11 can end its transmission if it doesn’t have data to transmit of the appropriate channel access class. IEEE 802 requests that 3GPP RAN specify LAA Rel.14 to accommodate partial sub-frames of one OFDM symbol duration in order to minimize channel wastage resulting from coarse sub-frame granularity in LAA.

RAN1 response 6:

The choice of ending partial subframe is dependent on more factors than the amount of data that needs to be transmitted, such as the control information to be transmitted. However, RAN1 notes that the following rules have been defined in Section 5.7.2 of TS 36.300 for multiplexing of data traffic and that these multiplexing rules are applicable to ending partial subframes as well.

-	the transmission duration of the DL transmission burst shall not exceed the minimum duration needed to transmit all available buffered traffic corresponding to Channel Access Priority Class(es) ≤ P;

-	the transmission duration of the DL transmission burst shall not exceed the Maximum Channel Occupancy Time (Tmcot,p as defined in Table 15.1.1-1 of [6]) for Channel Access Priority Class P;

-	additional traffic corresponding to Channel Access Priority Class(s) > P may only be included in the DL transmission burst once no more data corresponding to Channel Access Priority Class ≤ P is available for transmission. In such cases, E-UTRAN should maximise occupancy of the remaining transmission resources in the DL transmission burst with this additional traffic.

RAN1 notes that it is already agreed that transmission duration shall not exceed the minimum duration needed to transmit all available buffered traffic corresponding to Channel Access Priority Class(es) <= P. RAN1 also notes that if traffic belonging to Channel Access Priority Class(es) <= P is exhausted, then additional traffic corresponding to lower channel access priority classes can be multiplexed to maximize occupancy of the remaining transmission resources.

IEEE recommendation 7: IEEE 802 requests minimum duration be defined in LAA Rel. 13 and a subframe of one OFDM symbol be defined in LAA Rel. 14

These requirements, for the most part, have been satisfied by the updates to LAA Rel. 13. However the value for “minimum duration needed to transmit” appears to be open to interpretation.

IEEE 802 requests that for LAA Rel. 13, the minimum duration be approximated to the next occurring (partial) sub-frame boundary (one of 3/6/9/10/11/12/14 OFDM symbols). Also, for future releases of LAA (starting with Rel. 14), 3GPP should define partial sub-frames with a finer granularity including the provision for a sub-frame with 1 OFDM symbol.

RAN1 response 7:

RAN1 notes that the choice of size of the partial subframes to be used is also a function of the specification impact for considering many different sizes, eNB and UE implementation complexity and also the incremental gains that can be obtained. RAN1 also notes that it is currently discussing a 2-symbol, 4-symbol and 1-slot shortened TTI for frame structure type 1 (FS1) and 1-slot TTI for frame structure type 2 (FS2) as part of Rel-14 work item on shortened TTI and processing time  and may extend the specification to frame structure type 3 (FS3) if and when the work item is approved in the RAN plenary (see [3] for a candidate proposal that was discussed in RAN-73).

IEEE recommendation 8: IEEE 802 requests that 3GPP ensure that the LAA Rel. 13 aligns with the agreement that was achieved at ETSI-BRAN (Table 8 section 4.2.7.3.2.4 of BRAN-0060015v009), where a maximum TXOP is 6 ms and this may be increased up to 8ms with a minimum pause of 100μs or up to 10 ms with a doubled contention window size.

RAN1 response 8:

RAN1 thanks IEEE 802 for the information provided on the current status of discussions about maximum channel occupancy duration in ETSI BRAN.

RAN1 notes that LAA specification is intended for global deployment and it would be the responsibility of the eNB to comply with any regional regulations in addition to the LAA specification. In addition, RAN1 may consider alignment with all aspects of the ETSI BRAN specification including MCOT limits if there is consensus to do so.

IEEE recommendation 9: IEEE 802 suggests resolution of the channel access contention window issue be postponed until further tests and simulations have been completed

IEEE 802’s liaison statement to 3GPP RAN1 dated 18 March 2016 (IEEE 802.19-16-0037-09-0000-laa-comments.pdf) included the request that “3GPP explain and justify the selection of the 80% threshold for Z, and particularly why this value does not have an adverse effect on neighboring 802.11 devices.”

3GPP RAN1’s response to this comment was included in 3GPP RAN1’s Liaison Statement dated 7 June 2016 (R1-166041). The response noted that “It is RAN1’s understanding that the equivalent value of Z used by IEEE 802.11 is 100%”. IEEE 802 believes this comparison is invalid. 802.11 has no equivalent to Z. The lack of an immediate ACK always causes the contention window to expand immediately. In contrast, LAA ACKs/NACKs affect the contention window more than 4ms after possible collisions.

It is currently unproven whether contention style access between contending stations that have different contention window update delays is effective. Therefore, IEEE 802 recommends that extensive simulation and testing of LAA and 802.11 coexistence be conducted to determine whether fair sharing of the channel actually occurs in typical medium to high congestion environments. IEEE 802 is committed to work cooperatively with 3GPP RAN and other organizations in all necessary testing and simulation work.

RAN1 response 9:

IEEE Observation: IEEE 802 believes this comparison is invalid. 802.11 has no equivalent to Z.

RAN1 notes that in 802.11, an ACK message is sent by the responding device as long as at least one of the A-MPDUs out of several that are transmitted is delivered successfully. The transmitting station upon receipt of this message can reset the window size to a minimum. RAN1 considers that an A-MPDU has similar characteristics to a series of LTE TBs in a DL transmission burst in that each TB is self-decodable and its decoding success or failure is acknowledged individually in the ACK message. From this point of view, the equivalent value of Z used by IEEE 802.11 is 100% as only the failure to decode all the scheduled A-MPDUs is considered as a transmission failure for the purpose of contention window adjustment. However, RAN1 acknowledges that due to the delay in HARQ-ACK reception at the eNB, all transmission bursts may not result in a CW update as in IEEE 802.11. This would occur only if the eNB transmits small bursts of data back to back and hence would happen rarely in practice.

IEEE Observation: The lack of an immediate ACK always causes the contention window to expand immediately. In contrast, LAA ACKs/NACKs affect the contention window more than 4ms after possible collisions.

RAN1 notes that IEEE 802.11 packets can be up to 6ms or more in practice (up to 10ms based on the current ETSI BRAN draft [2]) in duration and the acknowledgement of the transmission is only received at the very end of this transmission even if a collision happens at the very beginning. RAN1 thus notes that the timeline for receiving feedback in 802.11 is a function of the packet duration and in several cases would be a few milliseconds after a collision has occurred. Furthermore, RAN1 also notes that as part of a Rel-14 work item on shortened TTI and processing time [1], the minimum latency between the DL PDSCH and DL HARQ feedback for legacy 1ms TTI operation is to be reduced from the current 4ms for all frame structures (FS) including FS3 used for LAA. This would reduce the time between DL transmission and reception of ACK message from the current levels. Finally, while RAN1 recognizes that IEEE 802.11 has other mechanisms such as RTS/CTS to provide immediate feedback on a possible collision, RAN1 also understands that this is an optional mechanism and may not be used always.

IEEE Observation: It is currently unproven whether contention style access between contending stations that have different contention window update delays is effective. Therefore, IEEE 802 recommends that extensive simulation and testing of LAA and 802.11 coexistence be conducted to determine whether fair sharing of the channel actually occurs in typical medium to high congestion environments. IEEE 802 is committed to work cooperatively with 3GPP RAN and other organizations in all necessary testing and simulation work.

RAN1 notes that the delays in reporting the ACK have been a standard assumption for all LAA simulations and no coexistence issues have been identified specifically due to this delay including when IEEE 802.11 RTS/CTS was used. While RAN1 thanks IEEE for its commitment to cooperating with 3GPP, RAN1 believes such extensive simulation and testing is not necessary to resolve this particular issue.

IEEE recommendation 10: IEEE 802 thanks 3GPP RAN1 for its clarification on CWp adjustment

IEEE 802 thanks 3GPP RAN1 for the clarification. It is now our understanding that at any time, LAA will be contending for the channel using the channel access engine of only one priority class. The determination of which channel access engine to use will be made by the scheduler before the channel access. This is unlike 802.11, which has four parallel channel access engines that contend with each other. For this reason, in LAA, HARQ based CW updates will happen together for all four priority classes so that all channel access engines (whether active or not) can react to channel congestion and error. The exception to this rules is the contention window reset after K attempts at CWMaxp for a given priority class p, which will happen only for the priority class that was used consecutively K times at CWMaxp. IEEE 802 requests that 3GPP make any necessary corrections to IEEE 802’s understanding on this issue.

RAN1 response 10:

RAN1 confirms IEEE 802’s understanding of the procedure for contention window adjustment.

IEEE recommendation 11: IEEE 802’s comment on quanta of channel sense has been resolved.

RAN1 response 11:

RAN1 notes the response from IEEE. .

IEEE recommendation 12-1: Most of, but not all, IEEE 802’s issues related to LAA Rel. 13’s backoff mechanism have been resolved

However, there is one aspect of comment 12 that remains unresolved. IEEE 802 noted that the way LAA Rel. 13 is defined, the access mechanism is more like ALOHA than slotted ALOHA in the cases where the next transmission is ready after the post backoff is complete, with the subsequent loss of efficiency that is associated with ALOHA style mechanism. This issue is most likely to have a measurable adverse affect at medium loads. At low loads, the LAA Rel. 13 mechanism is likely to result in lower delay. At high loads, LAA Rel. 13 transmissions will naturally occur on slot boundaries.

IEEE 802 suggests that LAA Rel. 13 is further refined to specify that transmissions normally occur on slot boundaries in all load scenarios, at least in the cases where LAA Rel. 13 uses 9us slots. This will align LAA Rel. 13 with 802.11 in most user environments. This alignment will maximise the use of slotted ALOHA style access and thus maximse efficiency to the benefit of all stakeholders. It may be acceptable for this change to be incorporated into LAA Rel. 14.

RAN1 response 12-1:

RAN1 notes that the eNB can maintain slot alignment if it is continuously monitoring the channel. Alternatively, if the eNB does not monitor the medium continuously after the post-backoff procedure (for example, if the eNB does not have data to transmit), the eNB has to check whether the channel is idle for a defer period just before transmission. The slot alignment in this case changes similar to the case where a device in idle mode starts monitoring the medium.

IEEE recommendation 12-2: IEEE 802 would appreciate clarification of a new backoff issue

An IEEE 802 member has noted an additional point that needs clarification/rewording in LAA Rel. 13 related to backoff. In 802.11, a station with a frame that becomes ready after a previous post transmission backoff is allowed to transmit at the next slot boundary only if the channel has been sensed to be idle continuously for a duration equal to the initial defer immediately preceding the transmission of the frame (see IEEE P802.11ac-2013 section 9.19.2.3, condition f).

IEEE 802 notes that 3GPP RAN1 has agreed to align LAA access behaviour to match that of 802.11. However, LAA Rel.13 is ambiguous on this point. In particular, 3GPP TS 36.213-V13.2.0 section 15.1.1 states:

If an eNB has not transmitted a transmission including PDSCH on a carrier on which LAA Scell(s) transmission(s) are performed after step 4 in the procedure above, the eNB may transmit a transmission including PDSCH on the carrier, if the channel is sensed to be idle at least in a slot duration Tsl when the eNB is ready to transmit PDSCH

The clause above leaves open the possibility that the eNB can transmit after completion of post-backoff, if the channel is sensed to be idle for only one slot when the eNB is ready to transmit, even if the channel has not been sensed to be idle for a continuous duration equal to the initial defer immediately before the transmission.

IEEE 802 requests that 3GPP RAN confirm intent of this clause and clarify/disambiguate the language in LAA Rel.13 or possibly LAA Rel. 14.

RAN1 response 12-2:

RAN1 notes that the following has been agreed in a Rel-13 CR in [4] in Aug 2016 which should resolve this issue.

If an eNB has not transmitted a transmission including PDSCH on a carrier on which LAA Scell(s) transmission(s) are performed after step 4 in the procedure above, the eNB may transmit a transmission including PDSCH on the carrier, if the channel is sensed to be idle at least in a slot duration Tsl when the eNB is ready to transmit PDSCH and if the channel has been sensed to be idle during all the slot durations of a defer duration Td immediately before this transmission. If the channel has not been sensed to be idle in a slot duration Tsl when the eNB first senses the channel after it is ready to transmit or if the channel has been sensed to be not idle during any of the slot durations of a defer duration Td immediately before this intended transmission, the eNB proceeds to step 1 after sensing the channel to be idle during the slot durations of a defer duration Td..

IEEE recommendation 13:

Explicitly defining support for PD-based channel access in a future release of LAA specification.

- IEEE 802 notes that support for both preamble detection and preamble transmission (e.g. using CTS-to-Self frames) by LAA devices, together with an alignment of the PD and ED thresholds used by LAA devices with those of 802.11 technologies, would be mutually beneficial for coexistence between deployed 802.11, 802.11ax and LAA devices. This approach would form the basis for the most efficient and fair spectrum sharing between IEEE 802 and 3GPP technologies.

RAN1 response 13:

RAN1 notes that there was extensive discussion on the various coexistence mechanisms including IEEE 802.11 preamble transmission and preamble detection by LAA eNBs. Considering all the pros and cons of the possible approaches, RAN1 had concluded at that time that adopting an ED threshold of -72dBm for a 23dBm transmitter was the best way forward to enable a technology neutral mechanism for coexistence in the 5GHz band. After further discussion, energy detection was considered as a viable technology neutral solution which can ensure fair coexistence with IEEE 802.11 devices and there was no consensus to consider alternative mechanisms for spectrum sharing between IEEE 802 and 3GPP technologies at this time.

IEEE recommendation 14:

IEEE 802 LMSC kindly requests RAN1 to consider the following:

- Indicating its interest in a continued dialog towards a future framework for efficient sharing of the 5 GHz band as described above [as in IEEE recommendation 13]

RAN1 response 14:

While RAN1 welcomes communication and continued dialog with IEEE 802 on coexistence between technologies sharing the 5GHz band, RAN1 respectfully defers this question to RAN plenary for further consideration.

2. Actions:

To IEEE 802 LMSC

No action is requested at this time.

## 3. References

[1] RP-161299, “New work item on shortened TTI and processing time for LTE”, from Ericsson at RAN plenary in Busan, Korea, June 13 – 16 2016.

[2] BRAN-0060015v014, “Draft of the harmonized European standard for 5GHz RLAN”.

[3] RP-161880, “New work item on Enhancements to LTE operation in the unlicensed spectrum” Ericsson, Nokia, Qualcomm, Intel.

[4] R1-168555, “CR on LAA post transmission backoff”, Broadcom Corporation.

4. Date of Next TSG-RAN WG1 Meetings:

TSG RAN WG1 Meeting 88	13 – 17 February 2017		Athens, Greece

TSG RAN WG1 Meeting 88bis	  3 – 7   April 2017		USA